Digital Drive Amplifier

Technical Background

Version 3.0; July 29, 2002

Introduction

In 2002, Sony will introduce a remarkable addition to the ES audio/video line.  It's a new type of component that incorporates an SACD/CD/DVD player and an A/V receiver on a single chassis.  By combining these capabilities, the AVD-S50ES and AVD-C70ES establish a new benchmark in both convenience and quality.  They target the customer who wants the maximum home theater performance without the maximum in equipment.

To build components that would combine all these functions and meet the rigorous standards of ES, Sony engineers needed to re-evaluate some of the fundamental assumptions of amplifier design.

The limits of conventional power amps

Conventional power amplifiers present a host of limitations that are so familiar that receiver engineers automatically work around them.  However, Sony's design program for the AVD-S50ES and AVD-C70ES required more than the typical work-around.  We needed to address these issues directly:

- Circuit complexity.  In the context of today's home theater receivers, the analog power amplifier is out of place.  You have digital source material processed through a digital preamplifier—only to be converted to analog prior to amplification.

- Heat generation.  The heat thrown off by conventional power output transistors is a central fact of amplifier and receiver design.  Conventional amplifiers and receivers can include heat sinks, fans, and chassis layouts that isolate the output transistors at the back or sides.  Heat is always bad for electronics.  And heat is detrimental to the SACD/CD/DVD players that Sony's new components were to include.  Sony sought a more comprehensive solution for the AVD-S50ES and AVD-C70ES.

- Crossover distortion.  Conventional power amplifiers use complementary pairs or sets of transistors to handle the top half and the bottom half of the waveform.  This can create crossover distortion, the solution to which is amplifier bias—and that means more heat!

- Thermal modulation distortion.  As the changing audio signal passes through the typical output transistors, it causes immediate changes in the transistors' temperature.  Unfortunately, the temperature changes affect the transistors' handling of audio signal.  This is thermal modulation distortion.  Left unchecked, it can degrade sound quality.

- Open-loop distortion.  Conventional amplifiers typically generate substantial distortion in "open-loop" mode.  That's why amplifiers correct this distortion with Negative Feedback (NFB).  However, NFB exposes the signal to Transient Intermodulation Distortion and other dynamic problems.

Obviously, conventional receivers can incorporate smart design solutions that overcome—or at least work around—these limitations.  But Sony's design program for the AVD-S50ES and C70ES demanded more.  Sony ES engineers sought to overcome these fundamental limitations of analog amplifiers.  And they accomplished their goal with digital technology.

The Digital Drive Amplifier:

Inspired by the Super Audio CD

For years, it's been evident that digital power amplifiers can solve many of the intrinsic problems of analog amps.  But no digital amp has ever met the stringent requirements of Sony ES engineers.  Until now.  The new Digital Drive Amplifier draws on 1-bit technologies that Sony originally developed for the Super Audio Compact Disc.  The result is a breakthrough in home theater component design.

Simplicity of design

In high fidelity components, the simplest solution is usually the best because it subjects the signal to the fewest distortion-causing processes.  Unfortunately, conventional A/V receivers are anything but simple.  After Digital Signal Processing (DSP), every signal needs to be converted back to analog, run through a Low Pass Filter (LPF), sent through an analog volume control and then amplified.

The conventional A/V receiver is anything but simple.  The signal must run through a gantlet of processes and stages.

The Sony Digital Drive Amplifier is dramatically different.  There is no line-level Digital-to-Analog (D/A) converter.  Instead, the amplifier accepts the output of the DSP stage directly.  The output of the Digital Drive Amplifier provides the wattage that drives the speakers.  In this way, the signal remains digital until the last possible instant.  In fact, the power amp itself works like a "power D/A converter."  The new power amp even includes the volume control function.

The Sony Digital Drive Amplifier dramatically simplifies receiver design.  And in high fidelity, simpler is better.

Principle of operation

In the Digital Drive Amplifier, all input signals, whether Direct Stream Digital™ (DSD™) signal or Pulse Code Modulation (PCM), are converted into a stream of 1-bit pulses.

Sony uses this 1-bit pulse stream to switch a pair of power output transistors on and off.  The resulting output has enough wattage to drive a loudspeaker, as you can see in the diagram below.  You can think of it as a "powered pulse."

The "powered pulses" have much in common with the Direct Stream Digital signal that Sony developed for Super Audio CD.  If you look carefully at the pulses, you'll see that where the audio waveform is positive, the pulses are mostly 1.  Where the audio waveform is negative, the pulses are mostly 0.  In this way, a 1-bit pulse stream can represent the audio signal.  This is much like a DSD signal.  As with a DSD signal, a Low Pass Filter (LPF) is all you need to recover the original audio signal.

In the diagram above, (A) represents the output power pulse stream.  This combines two components, the original audio signal (B) and a noise component (C).  The audio signal (B) looks smooth and continuous because the frequencies are low.  The noise component (C) looks abrupt and spiky because the frequencies are high.  The LPF effectively separates out the audio signal, for extremely accurate music reproduction.

You can see the action of the LPF in the frequency analysis inset chart at the lower right of the diagram.  The insert shows the music signal (B) as a rectangle.  These are low frequencies, which pass.  The blue lines show the characteristic of the LPF, which suppresses the noise component (C) on the right.  These are high frequencies, which do not pass.

The S-Master Process

While Sony's Digital Drive Amplifier is simple in principle, the fidelity of the output signal depends on getting each pulse exactly right.  That is, the leading and trailing edges of each pulse must have the right timing—and the height of each pulse must be carefully controlled.  This is comparable to the requirements for Super Audio CD playback.  So to accomplish these goals, Sony used technologies developed for our legendary SCD-1 Super Audio CD/CD player.

Sony's own CXD9634 Large Scale Integrated circuit (LSI) provides the S-Master processing at the heart of Sony's Digital Drive Amplifier.

The Digital Drive Amplifier consists of two stages: the S-Master process to establish the pulse train and the Pulse Height Volume control—the point at which the user sets the volume level.  The S-Master process is performed by the Sony CXD9634, a proprietary Sony Large Scale Integrated circuit (LSI).

The S-Master process incorporates three important technologies:

- Clean Data Cycle

- Synchronous Time Accuracy Controller (S-TACT)

- Complementary Pulse Length Modulation (C-PLM)

EMBED Word.Picture.8

Sony's Digital Drive Amplifier incorporates five powerful channels onto a sleek, low profile printed circuit board.  Gone are the bulky heat sinks of conventional amps.  Note the encapsulated S-Master module for each channel.

Clean Data Cycle

While digital signals are inherently resistant to noise and distortion, they are susceptible to time-base errors called jitter.  Jitter commonly enters the signal during recording, playback and transfer.  Precise pulse timing is crucial to the Digital Drive Amplifier.  For this reason, Sony uses powerful technology to suppress jitter.

The typical method of controlling jitter is Phase Locked Loop (PLL) clock regeneration.  While the method does a good job of controlling high-frequency jitter, Sony also required excellent control at the low frequencies.  That's why ES engineers developed the Clean Data Cycle, the first stage of the S-Master process.  Clean Data Cycle is a state-of-the-art technology that can regenerate the digital signal with time-axis accuracy equivalent to the original A/D converter at the recording studio.

Even if the amplitude of every digital sample is 100% accurate, time-axis jitter can distort the analog result (top).  Sony's Clean Data Cycle actually calculates the original sampling interval and applies the calculated timing to the signal (bottom).

Using a supremely accurate clock, the Clean Data Cycle examines thousands of input pulses at a time, calculates the correct sampling interval and applies the clean interval to the output data.  In this way, jitter is completely eliminated—and the integrity of the original musical signal is restored.  The result is a dramatic change in reproduction.  The original acoustic space is fully revealed—an important advantage whether you're listening to music or enjoying a DVD movie.

Synchronous Time Accuracy Controller (S-TACT)

To control any further possibility of time-base jitter at the output, Sony incorporates the Synchronous Time Accuracy Controller (S-TACT) circuitry we developed for the SCD-1 Super Audio CD player.  S-TACT effectively isolates the final pulse generator from the digital process by feeding the master clock directly to the pulse generator circuit.  This avoids noise in the clock signal and maintains accurate pulse timing at the output.

The Synchronous Time Accuracy Controller (S-TACT) maintains accurate pulse timing at the output.

Low-distortion C-PLM

Previous digital amplifiers have used a 1-bit technology called Pulse Width Modulation or PWM.  That is to say, those digital amplifiers varied the width of pulses.  Unfortunately, PWM tends to expose the signal to second-order harmonic distortion.  That's why the Digital Drive Amplifier uses a Sony exclusive technology called Complementary Pulse Length Modulation (C-PLM).  C-PLM effectively controls the distortion, maintaining the integrity of the musical signal.

Pulse Height Volume Control

After the S-Master circuit, the power pulse signal passes to a Pulse Height volume control—the place at which user volume adjustments are executed.  Most digital volume controls adjust the volume by multiplying the digital samples by a coefficient between zero and one.  For example, to achieve a volume setting 6 dB below maximum, you can multiply each sample by 0.5.  This yields accurate results, but it does sacrifice some detail at the least significant bit.  Sony demanded more.

The full power pulse (A) represents the maximum setting of the volume control.  To turn the volume down 6 dB, the receiver cuts the voltage to the power pulse generator in half (B).

The Pulse Height Volume control adjusts the volume by adjusting the regulator that supplies voltage to the power pulse generator.  Because this method does not modify or reshape the original digital samples, there's no loss of information, no loss of detail.  Sound quality is maintained from very low volume settings like -50 dB all the way to maximum.

Powerful benefits

Armed with S-Master and Pulse Height Volume technology, the Digital Drive Amplifier stands apart.  It delivers high power output (up to 120 watts per channel x5, 6 ohms, 20 to 20,000 Hz, 0.7%), and exceptional sound quality.  Yet it's small enough to occupy the same chassis with a tuner, complete DSP preamplifier and five-disc SACD/CD/DVD changer!  In addition, the Digital Drive Amplifier delivers several subsidiary benefits:

- No crossover distortion

- Reduced thermal modulation

- Superb open-loop performance

No crossover distortion

Conventional power amplifiers use separate transistors to reproduce the upper and lower halves of the waveform.  Crossover distortion occurs when one power transistor switches off and the other switches on as the audio waveform "crosses over" the zero point between negative and positive.  This distortion is particularly bothersome because it remains constant as the signal gets softer.  The distortion is more audible during the quiet passages of music or movies.  Conventional amplifiers combat this distortion with the output transistor bias current of Class A/B designs.  Ironically, this bias current generates even more heat.  So in fighting one problem, designers cause another.

Class B amplifiers introduce a "kink" in the audio waveform where the wave passes between positive and negative.  Treating this crossover distortion is a major goal of conventional amplifier design.

Because the Digital Drive Amplifier uses pulse density, switching glitches get removed from the music by the low pass filter.  The system is immune to crossover distortion, even when the music or movie sound track is particularly soft.

Reduced thermal modulation distortion

Analog power amp output transistors create the shape of the audio waveform directly.  Unwanted variation in the waveform is heard directly as distortion.  For example, thermal modulation distortion occurs when the changes in the output signal cause heating or cooling in the transistors.  The temperature changes actually alter the performance of the transistor, and this affects the sound.

The audio signal can cause complex—and thoroughly unwanted—changes in the temperature and performance of the output transistors.  The impact on sound is called thermal modulation distortion.

In dramatic contrast, the Digital Drive Amplifier does not generate substantial heat.  So thermal effects are minimized at the source.  Any small thermal effects that might persist are minimized because digital amplifier output transistors do not "shape" the output waveform in the same way as analog output transistors must do.  With the Digital Drive Amplifier, thermal modulation distortion is no longer a concern.

Superb open-loop performance

Conventional amplifiers typically generate substantial distortion in "open-loop" mode.  That's why these amps use Negative Feedback (NFB).  NFB constantly compares the output with the input, identifying open-loop distortion and applying an equal-but-opposite correction signal at the input.  NFB is a powerful tool for limiting distortion, and NFB explains the low distortion specifications common today.  However, NFB exposes the signal to Transient Intermodulation Distortion and other dynamic problems.

In contrast, the Sony Digital Drive Amplifier achieves excellent fidelity without any negative feedback at all!  Distortion remains low without any sacrifice in transient and dynamic characteristics.  Music comes alive.  And movie sound effects like car crashes and gunshots retain all their original impact.

Frame and Plate Construction

The AVD-C70ES presents a special challenge in chassis design.  A 5-disc carousel changer requires large, clear space for the carousel disc tray, making it difficult to create a stable mounting for the amplifier and power supply circuitry and to check the spread of vibration.

Vibration is the enemy of CD, DVD and Super Audio CD players for two powerful reasons.  First, vibration in the disc or optical pickup triggers unwanted operation in the tracking servos.  This can radiate spurious noise throughout the chassis.  And this radiation occurs in exactly the wrong place—near the sensitive low-level optical pickup preamplifier.  To make matters worse, vibration can also cause subtle distortions in the audio circuitry.  Vibration can have tiny "microphonic" effects on capacitor values and point-to-point wiring.

The AVD-C70ES meets this challenge with the Frame and Plate (FP) chassis.  This matches the outer frame with an intermediate plate to create a two-layer structure.  The disc carousel occupies the lower layer while the A/V receiver has a stable, isolated space on the top.  Sony engineers were free to pursue the ideal layout for the mechanism, power supply and the audio circuit block.  Of course, the intermediate metal plate also works as a reinforcement, strengthening the entire chassis, blocking resonance and shielding the audio circuitry from radiated noise from the motor.

The Frame and Plate (FP) construction of the AVD-C70ES gives ample room to both the A/V receiver and the five-disc changer while it adds structural rigidity and blocks radiated noise.

Conclusion

Beginning with the PCM-701ES digital processor in 1982, Sony ES engineers have been extending the capabilities of digital audio.  Their achievements include the digital Compact Disc player, the Digital Audio Tape recorder, the digital preamplifier, the digital MiniDisc recorder, the digital Super Audio CD player and the digital CD recorder.  The Digital Drive Amplifier represents one more milestone in the digital march of progress.

Features and specifications

|  | AVD-S50ES | AVD-C70ES |
| --- | --- | --- |
| Power output (8 ohms, 20 - 20,000 Hz, 0.7% THD) | 80 watts x5 | 100 watts x5 |
| Power output (6 ohms, 20 - 20,000 Hz, 0.7% THD) | 100 watts x5 | 120 watts x5 |
| Disc Capacity | One | Five |
| SACD Playback | Multi-channel | Multi-channel |
| DVD-Video Playback | Yes | Yes |
| DVD Progressive Scan | Yes | Yes |
| CD/CD-R/W Playback | Yes | Yes |
| CD MP3 Playback | Yes | Yes |
| Dolby Digital® decoder | Yes | Yes |
| Dolby® Pro Logic® II decoder | Yes | Yes |
| dts® decoder | Yes | Yes |
| Digital Cinema Sound™ Cinema Studio EX modes | A/B/C | A/B/C |
| Analog Line Audio | 3 inputs/1 output | 3 inputs/1 output |
| Optical Digital Audio | 2 inputs/1 output | 2 inputs/1 output |
| Coaxial Digital Audio | 1 input | 1 input |
| Component Video | 1 output | 1 input/1 output |
| S-Video | 3 inputs/2 outputs | 3 inputs/2 outputs |
| Subwoofer Line Output | Yes | Yes |
| FM preset stations | 20 | 20 |
| AM preset stations | 10 | 10 |
| Remote Control | RM-SP50 preprogrammed | RM-CL70 preprogrammed LCD |

Sony Electronics Inc.

1 Sony Drive, Park Ridge, NJ 07656

www.sel.sony.com

© 2002 Sony Electronics Inc.  All rights reserved.  Reproduction in whole or in part without written permission is prohibited.  Features and specifciations are subject to change without notice.  Non-metric weights and measures are approximate  Sony, Digital Cinema Sound, Direct Stream Digital and DSD are trademarks of Sony.  Dolby, Dolby Digital and Pro Logic are trademarks of Dolby Laboratories Licensing Corporation.  dts is a trademark of Digital Theater Systems Corporation.  All other trademarks are property of their respective owners.