## 1 About Net MD

Q1 What is Net MD?

A1. In addition to all the usual functions of an MD Walkman, Net MD allows you to easily store digital audio files (e.g., audio CDs, MP3, WAV, Windows Media format files) on your computer and transfer the files to the MiniDisc recorder using the supplied "SonicStage" application software. For a more detailed explanation of the terminology, please click here. http://www.openmg.com/

Q2 What are the functional differences?

A2

## 2 About System requirements

Q1 What are the system requirements?

A1. You will need a personal computer whose operating environment meets at least the following requirements; however, we do not guarantee that the software will run on all personal computers that satisfy this use environment

Computer:IBM PC/AT or compatible

CPU:Pentium II 400MHz or higher (Pentium lll 450MHz or higher is recommended)

Hard disk drive space :More than 120MB

RAM:64MB or higher (128MB or higher is recommended)

Others:CD-ROM drive (capable of digital playback by WDM)

Sound Board

USB port (supports USB 2.0 Full Speed (previously USB 1.1))

OS:Manufacturer-installed

Windows XP Home Edition / Windows XP Professional / Windows Millennium Edition / Windows 2000 Professional / Windows 98 Second Edition

Display:High Color (16bit) or greater, 800 x 480 dots or more (800 x 600 dots or more is recommended)

Others:Internet access for web registration

Windows Media Player (version 7.0 or higher for playing WMA files)

The following systems are not supported.

Macintosh

Windows XP versions other than Home Edition or Professional

Windows 2000 versions other than Professional

Windows 98 versions other than Second Edition

Windows NT

Windows 95

Personally assembled PCs or operating systems

An environment that is an upgrade of the original manufacturer-installed operating system

Multi-boot environment

Multi-monitor environment

Note:

We do not ensure trouble-free operation of the system suspend, sleep, or hibernation function on all computers.

## 3 About Connectivity and Recognition

Q1 How do I actually go about connecting my Net MD to my personal computer? What are the steps involved?

A1 Load the supplied SonicStage Software CD-ROM into your personal computer and then install both the SonicStage software and the Net MD driver onto the computer's hard drive.

2. Connect the Net MD to your computer via the supplied special USB cable. You cannot use an ordinary USB cable.

Q2 Can I use my Net MD if there is no USB ports on my personal computer?

A2. You can use it like a portable MD player, but you can't use it in its role as a "Net MD."

Q3 Can I use my Net MD via a USB hub?

A3. We do not guarantee success.

Q4 Can I use an extension USB cable with my Sony Net MD player?

A4. No.

Q5 What kinds of input/output terminals are there?

A5

Q6 Can I connect my Net MD to my stereo system?

A6. Yes you can, if your stereo system has a line input terminal. (only MZ-N10)

Plug a connection cable into the line output terminal of the Net MD to connect it to other audio devices using optional cable(RK-G129 or RK-G136).

After connection, press the jog dial.

Turn the jog dial repeatedly until "USEFUL" appears in the display, and then press to enter.

Turn the jog dial repeatedly until "AUDIO OUT" appears in the display, and then press to enter.

Turn the jog dial repeatedly until "LINE OUT" appears in the display, and then press to enter.

Note:

This Net MD does not have a Digital Output function.

Q7 What is the simple checkpoint on a stereo plug and a monaural plug?

Q8 Even though a Net MD device is connected to a computer, the Net MD is not recognized by the computer.

A8. Make sure that SonicStageVer.1.5 is installed correctly

Try to uninstall and then reinstall SonicStageVer.1.5

Make sure that SonisStageVer.1.5 is present in the task tray.

Make sure the device driver is correctly installed:

Connect the portable player to your computer.

Access Device Manager.

You may see an Unknown Devices entry, Other Devices or ! displayed in Device Manager - Click the entry and then click Uninstall (Or right-click the entry and then click Uninstall).

The following message is displayed

Warning: you are about to remove this device from your system (In Windows 2000 Professional, Windows XP Home Edition/Windows XP Professional the following message is displayed Warning: you are about to uninstall this device from your system) Click OK and remove the USB cable from your computer.

Check that there no longer any Unknown Devices entry, Other Devices or  ! displayed in Device Manager and then connect your portable player again.

Notes:

Even if your portable player driver has been installed, it may occur that it is incorrectly recognized by your computer. If this is the case, follow the on-screen instructions of the Add New Hardware Wizard (In Windows 2000 Professional, Windows XP Home Edition/Windows XP Professional Found new hardware is displayed- Similarly you must follow the on-screen instructions.

It may occur that the driver has not been installed or that you have connected your portable player to your computer before the driver has been installed:

Double-click the name of portable player or Unknown Devices, click Reinstall Driver and then follow the on-screen instructions of the Update driver wizard (In Windows 2000 Professional, Windows XP Home Edition/Windows XP Professional follow the on-screen instructions of the Hardware Update Wizard).

After the installation is finished, access Device Manager.

Double-click Universal Serial Bus Controllers  - If the name of portable player is present, the installation has been successful.

## 4 About MiniDisc

Q1 Do I need to use a special kind of MiniDisc when I record using SonicStage software?

A1. Use the same MiniDiscs for recording on that you have always used.

Q2 Can I playback a MiniDisc I have recorded with my Net MD in conventional MD recorder/player?

A2. Yes, you can. The recording method is the same. Note however that songs that have been recorded in MDLP mode cannot be played on a machine that is not compatible with MDLP. Please record your songs in SP Stereo mode.

## 5 About Recording (except NZ-NE410)

Q1 How long can I record in each mode?

A1.

Q2 Can I record from an MD player (or from a stereo component system that has an MD player) to another MD player?

A2. You can make analog recordings, but you cannot digitally record from one MD player to another. To make analog recordings, use an analog cable to connect the LINE OUT/AUX OUT/REC OUT/HEADPHONE OUT or other terminals of one MD player (or stereo component system that has an MD player) to the other MD player.

Q3 How do I record from a stereo component system, or a radio cassette player or amplifier or other device?

A3. You connect the LINE IN (OPT) terminal of your Net MD with the LINE OUT/AUX OUT/REC OUT or other output terminals of your radio cassette player, etc. (You cannot record from input terminals such as the LINE IN/AUX IN or REC IN.)

Q4 Does the Net MD have a built-in microphone?

A4. No. Please use a plug-in microphone (sold separately) if you wish to make a recording via a microphone. The Net MD's microphone input terminal takes a 3.5-mm stereo mini jack.

Q5 Is the date recorded on the MiniDisc recorded too?

A5. If you synchronize the clocks, the date will automatically be recorded on the MiniDisc when you record. Dates are not recorded on songs that you check-out from your personal computer.

Q6 Can I record MP3-format files on my Net MD?

A6. You cannot directly copy MP3 files. You need to use the SonicStage application to convert the file format into ATRAC3.

Note

VBR files are not supported.

Q7 Can I record from a device that does not have an optical digital output terminal?

A7. Digital recording is not possible, but you can make analog recordings if you connect your Net MD to the device's analog output terminals (LINE OUT/AUX OUT/REC OUT/CD OUT terminal, headphone terminal, etc.).

Q8 Can I make a digital recording from a source that has a sampling frequency of 48 kHz?

A8 Yes. The Net MD has a built-in sampling rate converter that will automatically adjust 32-, 44.1- or 48-kHz sampling frequencies to the Net MD's sampling frequency of 44.1 kHz.

Q9 How do I record from a stereo system, radio cassette player, MD/CD player or other device that does not have LINE OUT/AUX OUT/REC OUT or other output terminals?

A9. Connect your Net MD to the device's headphone terminal.

If the headphone terminal socket diameter is greater than 3.5 mm, (6.3-mm-diameter socket), use a PC-234S or a PC-234HS plug adapter.

If the headphone terminal socket diameter is greater than 3.5 mm, (6.3-mm-diameter socket), use a PC-234S or a PC-234HS plug adapter.

Q10 Can I make synchro recording if an analog line is used?

A10 It is not possible to make synchro recording if you use an analog line input.

To make synchro-recording, connect to digital equipment such as CD players, DVD players.

Q11 What is the difference between directly recording from a CD player and recording using the SonicStage software application?

A11 When you use SonicStage you can record at high speed; however, “TYPE-S” is not supported for the checked-out stereo music.

Q12 What is "TYPE-S"？

A12 "TYPE-S"is Digital Signal Processor which build in the high spec MD components of Sony.

This unit using DSP TYPE-S, signal processing is twice as good as last model. It is made very high quality sound as like MD stereo component.

TYPE-S works for Play mode and Record mode.

Q13 Can I check-in music data that I have recorded with my Net MD into the SonicStage application?

A13 No. Songs that you have recorded directly onto your Net MD from a CD player or music data that you have recorded via a microphone onto your Net MD cannot be checked in to SonicStage.

Q14 At least how many seconds are needed for the system to recognize that there is no sound?

A14 Approximately 2 seconds.

Q15 How do I record over the top of existing songs?

A15 (This answer is for MZ-N10)

Turn the jog dial repeatedly until "OPTION" appears in the display, and then press to the dial.

Turn the jog dial repeatedly until "REC-Posi" appears in the display, and then press to the dial.

Turn the jog dial repeatedly until" From Here" appears in the display, and then press to the dial.

Note:

* When the Group mode is ON, recording will add songs, regardless of the [REC-Posi] setting.

* If you select a group and then begin recording, the song will be recorded at the end of that group.

* If you record just after you change Discs or if you record after you press the END SEARCH button (goes to the end of the recorded part of the Disc), a new group will be created at the end of the Disc and the song will be recorded into this new group.

A15 (This answer is for MZ-NF610 and MZ-N510)

Press MENU and press          repeatedly until "OPTION" appears in the display and then press ENTER..

Press repeatedly until "R-Posi" appears in the display and then press ENTER.

Press       repeatedly until "FrHere" appears in the display and then press ENTER.

Note:

* When the Group mode is ON, recording will add songs, regardless of the [R-Posi] setting.

* If you select a group and then begin recording, the song will be recorded at the end of that group.

* If you record just after you change Discs or if you record after you press the END SEARCH button (goes to the end of the recorded part of the Disc), a new group will be created at the end of the Disc and the song will be recorded into this new group.

A15 (this answer is for MZ-NF810)

Press MENU and press      repeatedly until "OPTION" appears in the display and then press ENTER..

Press        repeatedly until "REC-Posi" appears in the display and then press ENTER.

Press        repeatedly until "From Here" appears in the display and then press ENTER.

Note:

* When the Group mode is ON, recording will add songs, regardless of the [REC-Posi] setting.

* If you select a group and then begin recording, the song will be recorded at the end of that group.

* If you record just after you change Discs or if you record after you press the END SEARCH button (goes to the end of the recorded part of the Disc), a new group will be created at the end of the Disc and the song will be recorded into this new group.

Q16 How do I begin recording at the end of the part that has already been recorded?

A16 (Answer for MZ-N10)

Turn the jog dial repeatedly until "OPTION" appears in the display, and then press to the dial.

Turn the jog dial repeatedly until "REC-Posi" appears in the display, and then press to the dial.

Turn the jog dial repeatedly until "From End" appears in the display, and then press to the dial.

Note:

* When the Group mode is ON, recording will add songs, regardless of the [REC-Posi] setting.

* If you select a group and then begin recording, the song will be recorded at the end of that group.

* If you record immediately after you change Discs or if you record after you press the END SEARCH button (goes to the end of the recorded part of the Disc), a new group will be created at the end of the Disc and the song will be recorded into this new group.

A16 (Answer for MZ-NF610 and MZ-N510)

Press MENU and press          repeatedly until "OPTION" appears in the display and then press ENTER..

Press        repeatedly until "R-Posi" appears in the display and then press ENTER.

Press        repeatedly until "FrEND" appears in the display and then press ENTER.

Note:

* When the Group mode is ON, recording will add songs, regardless of the [R-Posi] setting.

* If you select a group and then begin recording, the song will be recorded at the end of that group.

*If you record immediately after you change Discs or if you record after you press the END SEARCH button (goes to the end of the recorded part of the Disc), a new group will be created at the end of the Disc and the song will be recorded into this new group.

A16 (Answer for MZ-NF810)

Press MENU and press          repeatedly until "OPTION" appears in the display and then press ENTER..

Press        repeatedly until "REC-Posi" appears in the display and then press ENTER.

Press        repeatedly until "From End" appears in the display and then press ENTER.

Note:

* When the Group mode is ON, recording will add songs, regardless of the [REC-Posi] setting.

* If you select a group and then begin recording, the song will be recorded at the end of that group.

* If you record immediately after you change Discs or if you record after you press the END SEARCH button (goes to the end of the recorded part of the Disc), a new group will be created at the end of the Disc and the song will be recorded into this new group.

Q17 How to record from CD player or DVD video player using optical digital cable?

A17 Connect with this unit Line in terminal and CD player , DVD video player's optical line out jack.

## 6 About Playback

Q1 How do I connect my Net MD to a stereo component system, a radio cassette player, an amplifier or other device and play songs back through them?

A1 Connect your Net MD to the devices as shown in the illustrations below.

## 7 About MDLP

Q1 What is the MDLP (MiniDisc Long Play) mode?

A1 The MDLP function has two modes. The first mode is "LP2 stereo recording (playback)" under which a recording twice as long as standard can be made on the MiniDisc. The second mode is "LP4 stereo recording (playback)" under which a recording four times as long as standard can be made on the MiniDisc.

The LP2 stereo recording (playback) mode is for long recording and playback. LP4 stereo recording (playback) mode is more suitable for recording long meetings and preparing Discs to play in your car. You can certainly record/playback in standard stereo mode if you wish to. You can also record tracks in different MDLP modes on the same MiniDisc; however, songs that have been recorded in MDLP mode cannot be played on a machine that is not MDLP-mode-compatible.

Q2 Can I record in MDLP mode?

A2 Yes, however, songs that have been recorded in MDLP mode cannot be played on a machine that is not MDLP-mode-compatible.

Q3 Can I perform MDLP recording of line input analog signals from a cassette deck, FM radio or other device?

A3 Yes. You can make an MDLP recording of a signal from any source.

Q4Can I play back a MiniDisc on earlier machines that do not have an MDLP function?

A4 If you select Stereo when you check-out the digital sound data, you will be able to play your Disc on a machine that does not have an MDLP function.

## 8 About Operation

Q1 Can I set a track marker when either recording or playing back?

A1 Yes. Note however that you cannot track mark a song that has been checked out from a personal computer to a MiniDisc.

Q2 How to name the tracks, groups and Discs?

A2 While recording or complete recording, you can name tracks , groups and Discs using the recorder's character palette.

Maximum number of characters that can be input per Disc, alphanumeric characters and symbols only is about 1700 characters .

Note

If you enter the symbol"//"between the characters in group names such as "abc//def", you may be unable to use the group function.

## 9 About the Group function

Q1 What is the Group function?

A1 This function allows you to assign your songs on a single MiniDisc to specific music genres or artists. By selecting a certain group, you can play or edit the songs you have assigned to that group.

Q2 What do you mean by "Group setting"?

A2 It means assigning a newly recorded song to a certain group, or taking several songs that have been recorded one after the other on a Disc and sorting them out into groups, depending on their genre or album or other common characteristic. For further details of the procedure, please refer to the Operating Instructions.

In the example below, there are eight songs recorded on a single MiniDisc. Songs 1 to 3 are in the first group, songs 5 and 6 are in the second group.

"Group- -"

If the Group mode is ON, songs that are not in a group will be temporarily assigned to the last group.

Q3 How can I playback using the Group function?

A3 .While playback, press       a few times and make the mark        displayed.  The Playback with the group function will start from next song.

Q4 Is it possible to select a playback mode when the group mode is on?

A4 (Answer for MZ-N10)

You can select from various play modes, such as single play (1 Track) , shuffle play (Shuffle), program play (PGM). You can also select repeat play in any of play mode.

While the recorder is playing, press the jog dial. Turn the dial until 'PLAY MODE' flashes in the display. Turn the jog dial to select the play mode, and then press Enter.

A4 (Answer for MZ-NF610 and MZ-N510)

You can select from various play modes, such as single play (1 Track) , shuffle play (Shuffle), You can also select repeat play in any of play mode.

1. While the recorder is playing, press MENU.

2. Press      or         repeatedly until "P-MODE" flashes in the display, and then press ENTER again.

3. Press        or         repeatedly to select play mode, and then press ENTER.

A4 (Answer for MZ-NF810)

You can select from various play modes, such as single play (1 Track) , shuffle play (Shuffle), bookmark play, program play (PGM). You can also select repeat play in any of play mode.

1. While the recorder is playing, press P-MODE     repeatedly.

2. Each time you press the button the play mode changes.

3. Select the play mode you wish.

## 10 About Various other functions

Q1Does the display have a backlight function?

A1 This unit doesn't have backlight function.

## 11 About Batteries

Q1 When my Net MD is connected to my personal computer, can I run the Net MD on its rechargeable battery alone?

A1 We do not guarantee that your Net MD will operate solely under the power of its rechargeable battery. If the Net MD is in the middle of rewriting a Disc and its power runs out, the data will be lost. It is better to power your Net MD with its AC adapter when it is connected to your personal computer.

Q2 How long will batteries last?

Q3 How long does the battery need to be charged? (Except MZ-NE410)

A3 When the battery is discharged completely, it takes about 3.5 hours to be charged completely.

Q4 Can I use AA nickel-hydrogen rechargeable batteries in my Net MD?

A4 Yes - but you will not be able to recharge them unless you have a nickel-hydrogen rechargeable battery charger.

Q5 If I run my Net MD off rechargeable batteries as well as alkaline batteries, which will run down first?( only MZ-N10, MZ-NF810)

A5 The rechargeable bateries run down first. And then alkaline batteries.

## 12 Trouble shooting

Q1 While connecting with personal computer, this unit and remote controller switches do not work.

A1 While connecting with the personal computer, this unit and remote controller 's volume, ■, SOUND switch only can operate.

Q2 While I am making a digital recording from my portable CD player, my Net MD displays "NO SIGNAL." What's the matter?

A2 If your portable CD player doesn't have an optical digital output terminal, it can't digital record.(Except MZ-NE410)

And some portable CD player can't make optical digital output when the player use a batteries.

If you have an above problem please try it using AC power adopter then connect to the outlet.

Some portable CD player can't make digital recording if the function of protection against sound skipping (ESP) is on.

If you have an above problem please the function is off.

For further details of the operation, please refer to the Operating Instructions of the portable CD player.

If your player has the function of G-PROTECTION switch and it can select 1 or 2, please set switch 1.

Q3 Why does the message 'Tr Full' appear and recording stop even though there is enough empty space in a MiniDisc?

A3 It is a system limitation.

If a MiniDisc is repeatedly recorded and deleted, an audio file might be separated into some parts and then recorded. Even in this case, the MiniDisc searches separated data immediately and playback. in correct order. However, each separated data are dealt as each single track therefore when the number of data reaches 254 in a MiniDisc, the MiniDisc becomes non-recordable. If you wish to record more, please erase unwanted tracks.

Q4 Although I playback music that is in a Net MD Walkman on SonicStage, there is no sound.

A4 If you try to do this, no sound will come from your personal computer; however, you will be able to listen via the Net MD's headphones.

Q5 I can't play back music on my car stereo.

A5 Perhaps your MiniDisc songs have been recorded in LP mode. Devices that are not LP mode-compatible will not play back songs that have been recorded in LP mode.

Q6 The volume is very low when I play back a MiniDisc I recorded. Why?

A6 (Answer for MZ-N10)

Probably the recording level is low.

While recording, the recording level is automatically adjusted an appropriate level.But in need be, you can adjust the recording level manually .

1.While pressing this button press and slide the REC control ( Condition is pause)

2.press the jog dial.

3.Turn the jog dial、if you see [REC SET], push the jog dial.

4.Turn the jog dial、if you see [RecVolume], push the jog dial.

5.Turn the jog dial、if you see [ManualREC], push the jog dial.

6.Play the sound what you want to record.

7.Adjust the recording level using jog dial, meanwhile you check the display.

8.Press this button then stop the recording.

Note:

Left and right sound level is not separated and can't adjust separately.

After stop the recording , the recording level will be set up to the automatic adjust level.

A6 (Answer for MZ-NF610 and MZ-N510)

Probably the recording level is low.

While recording, the recording level is automatically adjusted an appropriate level.But in need be, you can adjust the recording level manually .

1.While pressing this button press REC TMARK ( Condition is pause)

2.Press MENU.

3.Press        or        until you see [R-SET], press ENTER.

4.Press        or        until you see [RecVol], press ENTER.

5.Press        or        until you see [Manual], press ENTER.

6.Play the sound what you want to record.

7.Adjust the recording level using         or meanwhile you check the display.

8. Press this button then stop the recording.

Note:

Left and right sound level is not separated and can't adjust separately.

After stop the recording , the recording level will be set up to the automatic adjust level.

A6 (Answer for MZ-NF810)

Probably the recording level is low.

While recording, the recording level is automatically adjusted an appropriate level.But in need be, you can adjust the recording level manually .

1.While pressing this button press and slide the REC control ( Condition is pause)

2.Press MENU.

3.Press         or          until you see [REC-SET], press ENTER.

4.Press         or          until you see [RecVolume], press ENTER.

5.Press         or          until you see [ManualREC], press ENTER.

6.Play the sound what you want to record.

7.Adjust the recording level using    or   or           meanwhile you check the display.

8.Press this button   then stop the recording.

Note:

Left and right sound level is not separated and can't adjust separately.

After stop the recording , the recording level will be set up to the automatic adjust level.

Q7 I can't erase a song.

A7 You cannot use the Net MD to erase a song that has been checked out from a personal computer to a MiniDisc.

Q8 Why did recording begin at the end of the Disc even though I selected [From Here] when I chose the [REC-Posi] setting? (except MZ-NE410)

A8 When the Group mode is ON, recording will add songs, regardless of the [REC-Posi] setting. If you select the group and then record, the song will be recording at the end of that group.

If you record immediately after you change Discs or if you record after you press the END SEARCH button (goes to the end of the recorded part of the Disc), a new group will be created at the end of the Disc and the song will be recorded into this new group.

Q9 When I play back or edit my LP4 recordings, I notice noise in the recording. Why is that?

A9 LP4 achieves long-playing stereo recording using special compression technology. Editing using the Divide function (that splits songs) or the Combine function (that joins songs together) may generate noise at the points where songs are cut or joined together. We recommend that you record in either SP (stereo recording) or LP2 mode if sound quality is critical.

Q10 While the group mode is on, the track number is reduced. what is happened?

A10 It can be shown only the songs which are in the selected group.

Q11 Why the battery is discharged even though I didn't use it.

A11 If you select the power mode for "QUICK", the power is always on even you don't use it.

Therefore the battery duration will be short. If battery level is low and power mode is "QUICK", it may run out without using it.If you have this problem please try to recharge again or change it to another.

This is a QA for Network Walkman. Please add this QA on the web site.

Q Is it possilble to use my Network Walkman in SonicStage Ver.1.5 bundled with Net MD Walkman?

A Yes. However you will not be able to use your Network Walkman unless you download the driver for USB connection and install it. (We will have sent you the driver by the middle of December.)