RCD-W2000ES CD Recorder

Technical Background

Version 1.0; April 24, 2003

Introduction

Sony co-invented the Compact Disc, invented MiniDisc and spearheaded Digital Audio Tape (DAT).  Over the years, Sony has been passionate about developing and advancing these products.  The showcase for Sony's latest and best digital audio technologies is ES, the Elevated Standard in audio and video.

The RCD-W2000ES incorporates an audiophile-grade five-disc CD changer and a superb CD-R/RW recorder with dubbing at 4x normal speed.  In developing the W2000ES, Sony examined, reexamined and overturned conventional approaches to A/D conversion, D/A conversion, power supply, electronic design, chassis layout and mechanical engineering.  The result is a component of consummate sound quality.

Super Bit Mapping® recording

When the Compact Disc was new, its 16-bit signal delivered the full quality of the professional digital recordings of the day.  But studio recorders have long since surpassed 16-bit resolution with 20 and even 24-bit performance.  As professional digital systems began to exceed the 16-bit word length of the Compact Disc, Sony engineers began to ask if it was possible to capture better-than-16-bit sound quality in a standard 16-bit medium.  The result of this exploration was Super Bit Mapping (SBM®) circuitry, which Sony first launched in the K-1203 professional processor.  SBM circuitry then made its audiophile debut on the DTC-2000ES DAT recorder in 1994.

Cornerstones of psychoacoustics, these are the Fletcher-Munson equal-loudness curves.  Each red curve represents tones that appear to sound equally loud, even though the sound pressure level (SPL) required to produce them varies considerably.  Looking at the lowest curve, a very soft perceived loudness of 10 phons requires a 1 kHz tone of 10 dB SPL.  To achieve the same apparent loudness, a 15 kHz tone requires about 25 dB SPL and a 20 Hz tone requires a whopping 75 dB SPL!  The conclusion?  The human ear is exquisitely sensitive to sound—including noise—in the middle frequencies, but relatively insensitive to sounds at the frequency extremes.

The SBM design takes advantage of the psychoacoustic fact that the human ear is far more sensitive to low-level noise in the mid-frequencies than in the frequency extremes.  So SBM circuitry uses sophisticated noise shaping to shift low-level noise out of the mid-range frequencies and into the range above 10,000 Hz, where it is relatively inaudible.  The overall noise energy remains the same as standard 16-bit recording.  But the audible performance is equivalent to 20-bit systems!

Theory of the SBM process.  Sony shifts the noise out of the middle frequencies, where it's most audible, and up into the higher frequencies, where it's less audible.  Even though the total noise energy does not change, audible noise is substantially reduced!

To perform the Super Bit Mapping process on analog mic and line inputs, the RCD-W2000ES must convert the input into a digital signal of higher than 16-bit resolution.  The recorder uses a pulse A/D converter and a decimation filter to convert the analog input into a 24-bit digital signal.  The recorder, like all CD players, uses 16-bit quantization.  The 8-bit difference represents more precise quantization—additional musical information and reduced noise.

Sony's Super Bit Mapping circuit uses noise shaping filters that mirror the frequency sensitivity of the human ear.

The SBM circuit uses noise shaping filters with a frequency response similar to that of the human ear.  This reduces noise in the audible range, while it feeds back and reintegrates four bits of low-level data that are normally lost in the conversion to 16-bit audio.  In this way, the RCD-W2000ES records spectacular sound from analog sources.  You'll hear 16-bit recordings with musical accuracy that approaches 20 bits of resolution.  Music sounds more lifelike, more vivid, more clearly audible against a background of impeccable silence.

There's more.  Because the Super Bit Mapping process is "single ended," no special playback circuitry is required.  This means that SBM encoding is fully compatible with any CD player that accepts CD-R/RW discs.  You'll get the benefit of the SBM process no matter what CD player you use!

Multi-Bit Delta-Sigma D/A Converter

In the 1980s, the overwhelming majority of CD players used multi-bit Digital-to-Analog (D/A) converters.  While these converters could offer superb dynamic range, they were susceptible to a distortion called low-level nonlinearity.  Multi-bit converters are prone to this distortion because the Most Significant Bit (MSB) represents 32,768 times the voltage of the Least Significant Bit (LSB).  So even slight errors in the MSB could overwhelm the musical signal at the zero cross, where the binary digits "flip" from 1111111111111111 to 0000000000000000.  These errors are generally masked by the music, when it is loud.  But when the music is soft, the errors can impart a "grit" or "hardness" to the music that university researchers found to be audible.

For this reason, technologists developed 1-bit D/A converters that bypassed the problem completely.  Significant among these 1-bit designs was Sony's own High Density Linear Converter™ circuit, which made its debut on the landmark CDP-X77ES in 1990.  The 1-bit D/As quickly became the dominant design for CD playback at all price levels.

While 1-bit designs continue to work well, they do have one characteristic problem of their own—large amounts of high-frequency noise.  Sony ES engineers have long used multi-order noise shaping filters to control that noise.  But the design program for the RCD-W2000ES demanded more.  For this CD recorder, Sony engineers employ a multi-bit Delta-Sigma D/A converter that combines the best of both worlds.  You get the superb dynamic range of multi-bit converters, with the exceptional linearity at low level of 1-bit designs.

Even when the music is extremely soft, the output of a 1-bit D/A converter cycles between plus and minus full scale, generating substantial high-frequency noise.  With its odd-level quantizer, the D/A converter of the RCD-W2000ES is considerably quieter, to reproduce music against a background of silence.

By their very nature, 1-bit converters always toggle between plus full scale and minus full scale, even when the music is extremely soft.  This switching generates substantial high-frequency noise.  In dramatic contrast, the RCD-W2000ES uses an odd-level quantizer.  When the music gets soft, the D/A converter output is mostly midscale zero code, with occasional plus and minus LSB steps.  So noise remains vanishingly low.  And music is reproduced against a background of silence.  Because the digits do not simultaneously flip from 1111111111111111 to 0000000000000000, the odd-level quantizer also guards against zero cross distortion, for excellent low-level linearity.  In this way, Sony delivers the best attributes of both 1-bit and multi-bit designs.

Sony's approach has additional benefits.  Idle-pattern tones—the persistent converter activity at zero signal—are minimized.  Clock jitter sensitivity is reduced.  And the need for analog post filtering is relaxed, because there's less noise to begin with.  For superior performance, Sony marries a low-power switched capacitor (SC) D/A converter to a hybrid post filter.

Twin R-Core Transformers

Power supply transformers take household 120-volt, 60-Hz alternating current and step it down to the voltages used by audio components.  Unfortunately, the transformer can degrade sound quality.  Alternating current in the transformer's coils of wire can cause the wires to vibrate and generate acoustic noise.  Electromagnetic fields can radiate from the transformer and induce 60 Hz hum in the audio circuits.

To control these problems, the RCD-W2000ES uses round or "R" Core power transformers.  Compared to the typical EI core transformers, the R-Core design offers a tighter, more consistent wire wrap, for silent, stable, low-radiation performance.

For the RCD-W2000ES, Sony has deployed two separate R-Core transformers.  The first supplies power to the analog circuitry while the second takes care of the servo and digital circuitry.  Separating the power supplies removes a potential path for mutual interference.

Twin R Core power transformers minimize vibration, radiated hum and noise.

To control even the subtle influences that a power supply can have over analog audio circuitry, the analog power supply features discrete components carefully selected for sound quality.  For example, Sony has chosen expensive electrolytic capacitors.

Circuit Topology

The circuit boards are carefully laid out to avoid interference between circuits.  For example, the power supply circuit, the audio circuit and the digital circuit are separated to limit mutual interference.

The topology of the circuit boards is carefully arranged to minimize mutual interference between circuits.

Balanced Construction

The RCD-W2000ES is the product of a comprehensive program to minimize the effects of air-borne, shelf-borne and even chassis-borne resonance.  A case in point is the chassis layout.  Sony located the CD changer and power supply on the left side of the unit, while the CD recorder and main circuit board are located on the right.  This balanced approach helps suppress chassis vibration.

Balanced construction helps suppress resonance. Here you see the CD changer on the left, with the twin R-Core power transformers at the top left.  The CD recorder is on the right with the main circuit board at the top right.

BASC Front Panel

The front panel of an audio component is a key element in the overall structure.  Not only must the panel resist resonance, it should also offer high-precision structural points for bolting on other chassis components.  While molded front panels are common today, Sony ES components often use aluminum front panels, prized for their rigidity, dimensional precision and anti-resonant properties.  The RCD-W2000ES goes beyond even this high standard with Sony's Brass Aluminum and Steel Construction (BASC) front panel.  The panel uses a brass guide on a steel base attached to an aluminum front.  Each type of metal has its own distinct vibrational characteristics.  When the metals are combined, the resonant vibrations in one metal tend to be damped out by the characteristics of the other two.  In this way, Sony's BASC front panel combines high strength, high precision and a remarkable resistance to vibration.

Sony's BASC front panel features a brass guide on a steel base attached to an aluminum front.

Off-Center Insulator Feet

By their very nature, equipment shelves have large, flat surfaces exposed to loudspeaker sound in the listening room.  For this reason, Sony takes special steps to prevent shelf-borne vibration from entering the chassis.  Sony's insulator feet are made from anti-resonant material.  And the screws used to attach these feet pass through off-center holes.

Centered holes tend to establish a single resonant frequency inside the foot, increasing the likelihood of transmitted vibration.  Off-center holes create a continuously varying radius from the hole to the edge.  This spreads resonance across a broad band of frequencies, further suppressing vibration.

Sony's Off Center Insulator feet are effective at blocking shelf-borne vibration.

Operating Features

As an audiophile CD player and a CD recorder on a single chassis, the RCD-W2000ES also offers a longer than usual list of operating features, some of which are unique to CD-R/RW recorders.

- Normal and High Speed Recording.  During normal-speed recording, you can monitor the music as you record.  The RCD-W2000ES can also record at 4x normal speed.  This means you can copy a 1-hour program in just 15 minutes.  Note that during high-speed recording, you cannot monitor the music as you record.

- Normal Syncro Recording automatically transfers a program of individual tracks from up to five CDs onto CD-R or CD-RW.  The recorder automatically switches into Record/Pause mode when each new track is accessed, then automatically switches back into Record mode when each new track is played.  After the recording session, the user must finalize the disc so that it can be played on other machines.

- Make-Disc Syncro Recording automatically records an entire CD onto CD-R/RW and automatically finalizes the disc when done.

- High Speed Syncro Recording.  Both the Normal and Make-Disc Syncro modes are available for high speed recording as well as normal speed.

- CD TEXT Information Entry.  In Make-Disc Syncro Recording, any CD TEXT information on the source disc is automatically transferred to the new disc.  In other cases, you can manually enter CD TEXT information: up to 40 tracks per disc with up to 70 characters per track.

- Direct Digital Recording.  You can record digital audio signals that use the CD-standard 44.1 kHz sampling rate directly onto CD-R/RW, without the need to convert the signals to analog and reconvert them to digital.  This enables superb digital "clones" that are bit-for-bit copies of the original.

- Dual Mode.  You can use Deck A and Deck B of the RCD-W2000ES to simultaneously feed two amplifiers in different rooms.  In Dual Mode, the player sends the signal from Deck A to the Output 1 jacks, while the Output 2 jacks get the signal from Deck B.  When the Dual Mode is off, the sound of either deck that is currently playing (A or B) is sent to both Output 1 and Output 2 jacks.

- MP3 Playback.  Popular for Internet downloads and extended playing time when ripping CDs, MP3 (MPEG 1 Audio Layer-3) is a standard technology and format for compressing audio data.  MP3 technology can compress a CD signal to about 1/10 of its original size, by reducing the bits devoted to tones you can't hear, while conserving bits for tones you can hear.  Deck A of the RCD-W2000ES can play MP3 files recorded on CD-ROM’s, CD-Rs (recordable CDs), and CD-RWs (rewritable CDs), including discs recorded in Multi Session.

- Super Bit Mapping mode switch.  When you record from analog mic or line sources, you can engage Super Bit Mapping circuitry by means of a front panel switch.  Remember, SBM encoded discs are fully compatible with any CD player that accepts CD-R/RW discs.  Because the Super Bit Mapping process is "single ended," no special playback circuitry is required.  You'll get the benefit of the SBM process no matter what CD player you use.

- Recording Level Control.  Digital input levels can be controlled through the setup menu, while analog input levels can be adjusted via the front panel Rec Level control.

- Silent Pause.  This function automatically puts the recorder into Record/Pause mode when it encounters 30 seconds of silence.  So you won't ruin an entire disc by inadvertently leaving the W2000ES in the Record mode.  If the silence continues for three minutes after Silent Pause activates, the W2000ES will automatically go into the Stop mode.  The Silent Pause function is disabled during Syncro Recording.

- Auto Space.  The RCD-W2000ES can automatically insert two seconds of silence between tracks when you stop recording or when the built-in CD changer changes discs during Syncro Recording.

- Fader Function.  You can gradually fade the sound in at the beginning of a track or fade out at the end.  Fades can extend up to a duration of five seconds.

RCD-W2000ES Specifications

| Features | RCD-W2000ES |
| --- | --- |
| Drive Type | Dual drives: 5 CD changer + CD-R/RW recorder |
| High-Speed Dubbing | 4x speed |
| High Speed Finalize | Yes |
| Sampling Rate Converter | Yes |
| Analog Record Level Control | Yes |
| Digital Record Level Control | Yes |
| CD Syncro Recording | Yes |
| Super Bit Mapping Recording | Yes |
| CD TEXT Display | Yes |
| Dual Mode | Yes |
| MP3 Playback (Deck A) | Yes |
| Specifications | RCD-W2000ES |
| A/D Converter | 96 kHz, 24-bit |
| D/A Converter with Digital Filter | 192 kHz, 24-bit |
| Number of D/A Converter ICs | Two |
| Frequency Response | 5 Hz - 20 kHz +/-0.5 dB |
| Distortion Playback (1 kHz, 0 dB) Recording (1 kHz, 0 dB) | <0.004% <0.01% |
| Signal-to-Noise Ratio Playback Recording (analog input) | >100 dB >90 dB |
| Dynamic Range Playback Recording (analog input) | >100 dB >90 dB |
| Outputs Analog Output, Fixed Headphone Output Digital Output | 2 V, 47 kohms 28 mW, 32 ohms 2 coaxial, 2 TOSLINK optical |
| Inputs Analog Input Digital Input | 500 mV, 47 kohms 1 coaxial, 1 TOSLINK optical |
| Remote Control | RM-R52 |
| Power Requirements | 120 V, 60 Hz |
| Power Consumption | 29 W |
| Dimensions (W x H x D) | 17 x 4-3/8 x 15-3/4" (430 x 110 x 400 mm) |
| Weight | 17 lbs., 7 oz. (7.9 kg) |

Sony Electronics Inc.

1 Sony Drive, Park Ridge, NJ 07656

www.sel.sony.com

© 2003 Sony Electronics Inc.  All rights reserved.  Reproduction in whole or in part without written permission is prohibited.  Features and specifciations are subject to change without notice.  Non-metric weights and measures are approximate  Sony, High Density Linear Converter, SBM and Super Bit Mapping are trademarks of Sony.  All other trademarks are property of their respective owners.